Venus, 19 August 2006 06:00-35 UT

| Name | Arnaud van Kranenburg |  |  |  |
| --- | --- | --- | --- | --- |
| Location | 54º51’ N 4º20’ E Vlaardingen, the Netherlands At sea level |  |  |  |
| Weather conditions | Seeing poor to fair Transparency fair, cloudy/foggy at times Relatively fast high winds, cloudys moving very quickly |  |  |  |
| Telescope | Celestron C9.25 with 2x Ultima barlow |  |  |  |
| Camera type | Lumenera SKYnyx 2-0 |  |  |  |
| Filter type | Violet bandpass filter at 390 nm Astronomik RGB filterset |  |  |  |
| Settings | Violet | Red | Green | Blue |
| effective focal length: gain shutter speed framerate arc/pixel resolution dynamyc range | f/36 20.1 x 33.3 ms 22 fps 0.18 12 bit | f/36 1.0 5.4 ms 60 fps 0.18 12 bit | f/36 1.0 5.4 ms 60 fps 0.18 12 bit | f/36 1.0 5.4 ms 60 fps 0.18 12 bit |
| Processing | 2 Sets of violet frames were aligned and stacked in Registax 3. A total of 8000 frames out of 16500 were selected, prefiltered and stacked. Afterwards the stacked images were wavelet enhanced and level corrected in Photshop. Finally a slicht noise reduction was applied. RGB, ca 800 out of 1500 frames were selected for each channel and aligned and stacked in Registax 3. They were slightly wavelet enhanced and a mild noise reduction was applied |  |  |  |
| Orientation | South is up |  |  |  |
| Local conditions Height above horizon: Venus diameter (arcsec) | Ca. 28º 10.4” |  |  |  |